Natural convection coefficient was calculated using a resistor surface temperature of 450K and an ambient air temperature of 298 K.

Geometry:

Length: 49 E-3 m

Diameter:14.8 E-3 m

Surface area:1.33 E-3 m^2

The following relations were used.

fundamentals of heat and mass transfer,7th edition, Bergman, Lavine, Incorpera, Dewitt, pg.611

where

L= characteristic length=Surface Areaperimeter=0.146096 m

υ=μρ

At 450 K…

α = 47.2m2s see μcitation

ρ=0.7788kgm3 see μcitation

μ=2.504 E-5 kgm*s fundamentals of heat and mass transfer,7th edition, Bergman, Lavine, Incorpera, Dewitt, pg.995

β=2.22E-3 K-1 https://www.engineeringtoolbox.com/air-density-specific-weight-d_600.html

Pr=0.7 https://www.engineeringtoolbox.com/air-prandtl-number-viscosity-heat-capacity-thermal-conductivity-d_2009.html

Ra =828805 (unitless) (calculated with above equation)

fundamentals of heat and mass transfer,7th edition, Bergman, Lavine, Incorpera, Dewitt, pg.610

kair=36.76E-3 WmKhttps://www.engineeringtoolbox.com/air-properties-viscosity-conductivity-heat-capacity-d_1509.html?v

hNatural convection=Nu*kairLchariteristic=4.1Wm2K

Using fourier’s Law

keffective= -QA(ΔxΔT)

Area in fourier’s law equals the footprint of the resistor on the PCB.

Width = 14.8 E-3 m

Length = 49.8 E-3 m

Area = 7.37 E-4 m^2

Applied Voltage =12 volts

P=V2R=14410=14.4 W

Working from the different pictures we took…

Measured area on circuit board dimensions

Length of rectangle = 50.35 mm

Width = 31.66 mm

Qnatural=(4.1Wm2K)(1.33 E-3m2)(450.3-298) =.8305W

Applied heat:

Q=14.4W-.83W=13.57W

I only used part of the data in order to get a good fit line to extract dT/dx.

Each temperature line taken from the rectangle is approximately 6 mm apart.

Experimental K values are at the bottom of each table.

|  | Distance (m) | T1 | T9 | T17 | T24 | T31 | T38 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4.07E-02 | 339.238 | 345.537 | 346.427 | 345.963 | 344.513 | 342.323 |
|  | 3.96E-02 | 340.379 | 346.565 | 347.057 | 346.831 | 345.497 | 343.271 |
|  | 3.86E-02 | 341.521 | 347.351 | 348.027 | 347.978 | 346.427 | 344.263 |
|  | 3.75E-02 | 342.312 | 348.154 | 348.826 | 348.777 | 347.371 | 345.269 |
|  | 3.64E-02 | 343.823 | 348.913 | 349.717 | 349.901 | 348.349 | 346.091 |
|  | 3.54E-02 | 345.001 | 350.036 | 350.854 | 350.777 | 349.369 | 347.293 |
|  | 3.43E-02 | 346.24 | 351.123 | 351.754 | 351.735 | 350.267 | 348.319 |
|  | 3.32E-02 | 347.557 | 352.108 | 352.821 | 352.84 | 351.582 | 349.456 |
|  | 3.21E-02 | 348.553 | 353.039 | 354.06 | 354.05 | 352.735 | 350.816 |
|  | 3.11E-02 | 349.969 | 354.812 | 355.412 | 355.468 | 354.032 | 351.917 |
|  | 3.00E-02 | 351.535 | 355.757 | 356.725 | 356.725 | 355.449 | 353.266 |
|  | 2.89E-02 | 352.745 | 357.087 | 357.789 | 358.093 | 356.781 | 354.248 |
|  | 2.79E-02 | 354.145 | 358.13 | 358.92 | 359.442 | 357.918 | 355.627 |
|  | 2.68E-02 | 355.608 | 359.259 | 360.154 | 360.454 | 359.25 | 357.253 |
|  | 2.57E-02 | 357.013 | 360.626 | 361.496 | 361.713 | 360.59 | 358.415 |
|  | 2.46E-02 | 358.333 | 361.992 | 362.784 | 363.053 | 362.082 | 360.126 |
|  | 2.36E-02 | 359.497 | 363.169 | 364.312 | 364.854 | 363.697 | 361.875 |
|  | 2.25E-02 | 361.161 | 364.614 | 365.933 | 366.224 | 365.323 | 363.303 |
|  | 2.14E-02 | 362.909 | 366.233 | 367.19 | 367.951 | 366.804 | 365.164 |
|  | 2.04E-02 | 364.49 | 367.75 | 368.761 | 369.464 | 368.813 | 366.866 |
|  | 1.93E-02 | 366.092 | 369.447 | 370.509 | 371.462 | 370.569 | 368.726 |
|  | 1.82E-02 | 367.846 | 370.982 | 372.343 | 373.194 | 372.454 | 370.664 |
|  | 1.71E-02 | 369.559 | 372.735 | 373.864 | 374.784 | 374.202 | 372.735 |
|  | 1.61E-02 | 371.471 | 374.481 | 376.027 | 376.83 | 376.454 | 375.146 |
|  | 1.50E-02 | 373.389 | 376.446 | 377.894 | 378.623 | 378.441 | 377.146 |
|  | 1.39E-02 | 375.272 | 378.474 | 379.727 | 380.826 | 380.572 | 379.711 |
|  | 1.29E-02 | 377.005 | 380.212 | 381.805 | 383.014 | 382.933 | 382.146 |
|  | 1.18E-02 | 379.151 | 382.285 | 383.878 | 385.187 | 385.451 | 384.875 |
|  | 1.07E-02 | 381.169 | 384.626 | 386.425 | 387.774 | 387.924 | 387.798 |
|  | 9.64E-03 | 383.789 | 386.664 | 388.556 | 389.915 | 390.603 | 390.658 |
| dt/dx |  | -1403 | -1302 | -1337 | -1389 | -1447 | -1504 |
| K W/m*K |  | 13.12365149 | 14.14169204 | 13.77149068 | 13.25592731 | 12.7245909 | 12.24234245 |

Line 3

Temperature of the resistor is less than 1K less than the previous picture. This will change the convective heat coefficient by less than our working sig figs. Using the same loss to natural convection as the previous figure.

Applied heat:

Q=14.4W-2.1W=12.3W

|  | Distance (meters) | Temperature (Kelvin) |
| --- | --- | --- |
|  | 0.00944064 | 391.468 |
|  | 1.05E-02 | 388.871 |
|  | 1.15E-02 | 386.512 |
|  | 0.01258752 | 384.152 |
|  | 1.36E-02 | 381.748 |
|  | 1.47E-02 | 379.464 |
|  | 0.0157344 | 377.313 |
|  | 1.68E-02 | 375.356 |
|  | 1.78E-02 | 373.279 |
|  | 0.01888128 | 371.471 |
|  | 1.99E-02 | 369.672 |
|  | 2.10E-02 | 367.82 |
|  | 0.02202816 | 366.101 |
|  | 2.31E-02 | 364.561 |
|  | 2.41E-02 | 362.981 |
|  | 0.02517504 | 361.306 |
|  | 2.62E-02 | 360.017 |
|  | 2.73E-02 | 358.618 |
|  | 0.02832192 | 357.327 |
|  | 2.94E-02 | 356.102 |
|  | 3.04E-02 | 354.624 |
|  | 0.0314688 | 353.399 |
|  | 3.25E-02 | 352.06 |
|  | 3.36E-02 | 351.056 |
|  | 0.03461568 | 350.055 |
|  | 3.57E-02 | 348.748 |
|  | 3.67E-02 | 347.978 |
|  | 0.03776256 | 346.831 |
|  | 3.88E-02 | 345.963 |
|  | 3.99E-02 | 345.1 |
|  | 0.04090944 | 344.263 |
|  | 4.20E-02 | 343.16 |
|  | 4.30E-02 | 342.505 |
|  | 0.04405632 | 341.725 |
|  | 4.51E-02 | 340.818 |
|  | 4.62E-02 | 340.44 |
| dt/dx |  | -1360 |
| k W/m*k |  | 13.53859047 |

Line 2

|  | Distance (meters) | Temperature (Kelvin) |
| --- | --- | --- |
|  | 0.06957108067 | 340.102 |
|  | 6.86E-02 | 340.859 |
|  | 6.76E-02 | 341.471 |
|  | 0.06667228567 | 342.363 |
|  | 6.57E-02 | 343.009 |
|  | 6.47E-02 | 343.993 |
|  | 0.06377349067 | 344.652 |
|  | 6.28E-02 | 345.676 |
|  | 6.18E-02 | 346.683 |
|  | 0.06087469567 | 347.322 |
|  | 5.99E-02 | 348.466 |
|  | 5.89E-02 | 349.408 |
|  | 0.05797590067 | 350.296 |
|  | 5.70E-02 | 351.563 |
|  | 5.60E-02 | 352.811 |
|  | 0.05507710567 | 353.871 |
|  | 5.41E-02 | 355.412 |
|  | 5.31E-02 | 356.679 |
|  | 0.05217831067 | 357.614 |
|  | 5.12E-02 | 358.884 |
|  | 5.02E-02 | 360.372 |
|  | 0.04927951567 | 361.568 |
|  | 4.83E-02 | 363.384 |
|  | 4.73E-02 | 364.907 |
|  | 0.04638072067 | 366.585 |
|  | 4.54E-02 | 368.256 |
|  | 4.44E-02 | 370.242 |
|  | 0.04348192567 | 371.89 |
|  | 4.25E-02 | 373.796 |
|  | 4.15E-02 | 376.044 |
|  | 0.04058313067 | 378.06 |
|  | 3.96E-02 | 380.105 |
|  | 3.87E-02 | 382.512 |
|  | 0.03768433567 | 384.69 |
|  | 3.67E-02 | 387.14 |
|  | 3.58E-02 | 389.664 |
|  | 0.03478554067 | 392.051 |
|  | 3.38E-02 | 394.979 |
|  | 3.29E-02 | 398.006 |
|  | 0.03188674567 | 400.836 |
|  | 3.09E-02 | 404.221 |
| dt/dx |  | -1.59E+03 |
| K W/mK |  | 1.16E+01 |

Line 4

|  | Distance (meters) | Temperature (Kelvin) |
| --- | --- | --- |
|  | 0.003213826667 | 405.08 |
|  | 4.29E-03 | 401.96 |
|  | 5.36E-03 | 398.767 |
|  | 0.006427651667 | 395.743 |
|  | 7.50E-03 | 393.026 |
|  | 8.57E-03 | 390.361 |
|  | 0.009641476667 | 387.853 |
|  | 1.07E-02 | 385.323 |
|  | 1.18E-02 | 383.03 |
|  | 0.01285530167 | 380.761 |
|  | 1.39E-02 | 378.648 |
|  | 1.50E-02 | 376.755 |
|  | 0.01606912667 | 374.54 |
|  | 1.71E-02 | 372.811 |
|  | 1.82E-02 | 370.827 |
|  | 0.01928295167 | 369.091 |
|  | 2.04E-02 | 367.26 |
|  | 2.14E-02 | 365.757 |
|  | 0.02249677667 | 364 |
|  | 2.36E-02 | 362.46 |
|  | 2.46E-02 | 360.935 |
|  | 0.02571060167 | 359.497 |
|  | 2.68E-02 | 358.379 |
|  | 2.79E-02 | 357.152 |
|  | 0.02892442667 | 355.841 |
|  | 3.00E-02 | 354.709 |
|  | 3.11E-02 | 353.437 |
|  | 0.03213825167 | 351.831 |
|  | 3.32E-02 | 350.701 |
|  | 3.43E-02 | 349.766 |
|  | 0.03535207667 | 348.787 |
|  | 3.64E-02 | 347.851 |
|  | 3.75E-02 | 346.821 |
|  | 0.03856590167 | 345.706 |
|  | 3.96E-02 | 345.2 |
|  | 4.07E-02 | 344.183 |
|  | 0.04177972667 | 343.662 |
|  | 4.29E-02 | 342.575 |
|  | 4.39E-02 | 341.958 |
|  | 0.04499355167 | 341.267 |
|  | 4.61E-02 | 340.358 |
|  | 4.71E-02 | 339.393 |
| dt/dx |  | -1427 |
| K W/mK |  | 12.90293135 |

New Data Set @ 6 Volts

Natural convection coefficient was calculated using a resistor surface temperature of 341K and an ambient air temperature of 298 K.

α=28.66 E-6 m2s

μ=204.235 E-7 kgm*s

ρ=1.02296 kgm3

Pr = 0.7117

β = 2.9482 E-3 K-1

υ=1.9965m2s

Ra=6.87198 E6

Nu=27.648

kair 341.6 K=0.016986 WmK

hnatural conv=3.21461 Wm2K

Qnatural=(3.21Wm2K)(1.33 E-3m2)(341.6-298) =0.186W

P=v2R=3610=3.6W

Papplied=3.6W-0.186W=3.4W

Area measurement

|  | Distance (meters) | t 8 | t 15 | t 22 | t 29 | t 34 |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0.03535207667 | 311.653 | 311.552 | 311.728 | 310.998 | 310.669 |
|  | 3.43E-02 | 312.104 | 312.054 | 311.941 | 311.552 | 311.074 |
|  | 3.32E-02 | 312.191 | 312.229 | 312.504 | 311.703 | 311.389 |
|  | 0.03213825167 | 312.566 | 312.666 | 312.753 | 312.191 | 312.029 |
|  | 3.11E-02 | 313.213 | 313.113 | 313.25 | 312.628 | 312.329 |
|  | 3.00E-02 | 313.485 | 313.349 | 313.473 | 313.25 | 312.653 |
|  | 0.02892442667 | 313.868 | 313.794 | 313.831 | 313.547 | 313.237 |
|  | 2.79E-02 | 314.041 | 314.275 | 314.287 | 313.992 | 313.461 |
|  | 2.68E-02 | 314.607 | 314.68 | 314.594 | 314.386 | 314.004 |
|  | 0.02571060167 | 314.766 | 314.987 | 315.207 | 314.668 | 314.484 |
|  | 2.46E-02 | 315.402 | 315.426 | 315.621 | 315.121 | 314.852 |
|  | 2.36E-02 | 315.694 | 315.986 | 315.937 | 315.597 | 315.28 |
|  | 0.02249677667 | 316.204 | 316.434 | 316.483 | 315.986 | 315.706 |
|  | 2.14E-02 | 316.676 | 316.857 | 317.038 | 316.507 | 316.156 |
|  | 2.04E-02 | 317.098 | 317.122 | 317.471 | 316.869 | 316.628 |
|  | 0.01928295167 | 317.375 | 317.711 | 317.687 | 317.759 | 317.122 |
|  | 1.82E-02 | 317.782 | 318.129 | 318.261 | 318.141 | 317.711 |
|  | 1.71E-02 | 318.392 | 318.701 | 318.892 | 318.701 | 318.237 |
|  | 0.01606912667 | 318.963 | 318.987 | 319.65 | 319.165 | 318.832 |
|  | 1.50E-02 | 319.591 | 319.838 | 319.803 | 319.756 | 319.401 |
|  | 1.39E-02 | 320.062 | 320.333 | 320.497 | 320.392 | 320.215 |
|  | 0.01285530167 | 320.603 | 320.837 | 321.165 | 321.375 | 320.954 |
|  | 1.18E-02 | 321.305 | 321.712 | 322.014 | 322.049 | 321.584 |
|  | 1.07E-02 | 321.887 | 322.072 | 322.582 | 322.698 | 322.339 |
|  | 0.009641476667 | 322.744 | 322.871 | 323.262 | 323.251 | 323.17 |
| dt/dx | -416 | -416 | -429 | -440 | -466 | -468 |
| K W/mK | 11.08965661 | 11.08965661 | 10.75360641 | 10.48476625 | 9.899779293 | 9.857472544 |

6 volt Trial 1

|  | distance (Meters) | Temperature (Kelvin) |
| --- | --- | --- |
|  | 2.06E-02 | 317.014 |
|  | 0.02157855 | 316.483 |
|  | 2.26E-02 | 316.144 |
|  | 2.36E-02 | 315.658 |
|  | 0.0246612 | 315.28 |
|  | 2.57E-02 | 314.876 |
|  | 2.67E-02 | 314.484 |
|  | 0.02774385 | 314.14 |
|  | 2.88E-02 | 313.683 |
|  | 2.98E-02 | 313.25 |
|  | 0.0308265 | 312.765 |
|  | 3.19E-02 | 312.341 |
|  | 3.29E-02 | 311.891 |
|  | 0.03390915 | 311.703 |
|  | 3.49E-02 | 311.338 |
|  | 3.60E-02 | 310.948 |
|  | 0.0369918 | 310.783 |
|  | 3.80E-02 | 310.556 |
|  | 3.90E-02 | 310.137 |
|  | 0.04007445 | 309.971 |
|  | 4.11E-02 | 309.627 |
|  | 4.21E-02 | 309.334 |
| dT/dx |  | -358 |
| K W/mK |  | 12.88630489 |

6 Volt trial 2

|  | Distance (meters) | Temperature (Kelvin) |
| --- | --- | --- |
|  | 0.01888128 | 317.627 |
|  | 1.99E-02 | 317.23 |
|  | 2.10E-02 | 316.905 |
|  | 0.02202816 | 316.216 |
|  | 2.31E-02 | 315.84 |
|  | 2.41E-02 | 315.341 |
|  | 0.02517504 | 315.048 |
|  | 2.62E-02 | 314.472 |
|  | 2.73E-02 | 314.189 |
|  | 0.02832192 | 313.856 |
|  | 2.94E-02 | 313.448 |
|  | 3.04E-02 | 312.89 |
|  | 0.0314688 | 312.716 |
|  | 3.25E-02 | 312.304 |
|  | 3.36E-02 | 311.791 |
|  | 0.03461568 | 311.628 |
|  | 3.57E-02 | 311.351 |
|  | 3.67E-02 | 310.973 |
|  | 0.03776256 | 310.669 |
|  | 3.88E-02 | 310.15 |
| dT/dx |  | -370 |
| K W/mK |  | 12.46837068 |

Thermal Conductivity Values w/ voltages separate

|  | Experimental Thermal Conductivity |  |
| --- | --- | --- |
|  | 12 Volt | 6 Volt |
|  | 13.12365149 | 11.08965661 |
|  | 14.14169204 | 11.08965661 |
|  | 13.77149068 | 10.75360641 |
|  | 13.25592731 | 10.48476625 |
|  | 12.7245909 | 9.899779293 |
|  | 12.24234245 | 9.857472544 |
|  | 13.53859047 | 12.88630489 |
|  | 1.16E+01 | 12.4683706 |
|  | 12.90293135 |  |
| average | 13.03346852 | 11.06620165 |
| standard deviation | 0.7809259678 | 1.104361736 |

Thermal Conductivity Values w/ voltages together

|  | Experimental Thermal Conductivity |
| --- | --- |
|  | 13.12365149 |
|  | 14.14169204 |
|  | 13.77149068 |
|  | 13.25592731 |
|  | 12.7245909 |
|  | 12.24234245 |
|  | 13.53859047 |
|  | 1.16E+01 |
|  | 12.90293135 |
|  | 13.03346852 |
|  | 11.08965661 |
|  | 11.08965661 |
|  | 10.75360641 |
|  | 10.48476625 |
|  | 9.899779293 |
|  | 9.857472544 |
|  | 12.88630489 |
|  | 12.4683706 |
| average | 12.15912769 |
| standard deviation | 1.342006495 |